CSE 4327 VLSI Design – Lab Final Viva Cheat Sheet

Know these first (fixes and gotchas)

- Mux 2:1 finishes at 60 ns (6 vectors), not 80. Mux 8:1: s=010 selects i2 (console: inputs 00000100, out=1). Both fixed in your corrected reports.

- Counter wraps 1111→0000 at 165 ns; your waveform only shows 0–80 ns. Re-run to 180 ns before the viva and be ready to show the wrap.

- JK first toggle takes effect at the 55 ns edge (inputs change at 50).

- Your + adders are behavioral, not ripple-carry. Test benches use $monitor (not self-checking) except Exp 7.

- Exp 7: confirm slack / cell count / power from your own rc output (the report's table values are marked “~”). Its SDC was carried over from Exp 6 (names rst/count). NOT report says DC sweep 0–2.0 V but plot ends ≈1.8 V.

Q1 & Q2 – Virtuoso: NOT, NAND, AND

- Setup: gpdk090, VDD=1.8 V, L=100 nm, Wp=720 nm, Wn=240 nm (3:1). vpulse 0→1.8 V, 20 ns / 10 ns width; NAND/AND input B 40 / 20 ns (all 4 combos); rise/fall 50 ps; transient 200 ns (NOT: 100 ns + DC sweep).

- Inverter: Vin=0 → PMOS on, Vout=VDD; Vin=1 → NMOS on, Vout=0. PMOS wider because hole mobility is lower → balanced rise/fall, VM≈VDD/2 (~0.9 V).

- NAND: 2 PMOS parallel (pull-up), 2 NMOS series (pull-down); output low only when A=B=1. NMOS stack is weaker → slower fall, VM shifts up.

- AND = NAND (PM0, PM1, NM0, NM1) + inverter (PM2, NM2); point to the internal node between stages. One CMOS stage always inverts. 6 transistors.

- VTC: DC sweep of Vin, plot Vout; VM is where Vin=Vout. Flow: library + tech → schematic → symbol → test bench → ADE L (Spectre, model lib, tran/DC).

- Swap PMOS/NMOS: becomes non-inverting with degraded levels (high ≈ VDD−Vtn, low ≈ |Vtp|).

Q3 – 4-to-1 MUX (Verilog)

- Yours = dataflow, one assign with nested ?:; say “dataflow”, not structural. Truth table: out = i[s].

assign out = s[1] ? (s[0]?i[3]:i[2]) : (s[0]?i[1]:i[0]);

- Three 2:1 muxes: two first-level (s[0]: i0/i1, i2/i3) + one second-level (s[1]). n:1 needs n−1 (8:1 → 7). Live edit:

mux21 m0(.i0(i[0]),.i1(i[1]),.s(s[0]),.out(w0));

mux21 m1(.i0(i[2]),.i1(i[3]),.s(s[0]),.out(w1));

mux21 m2(.i0(w0),.i1(w1),.s(s[1]),.out(out));

- #10 = 10 time units = 10 ns (timescale 1ns/1ps). Doubling it doubles each hold and the run (2:1 60→120, 4:1 80→160, 8:1 160→320 ns). Combinational: out changes in the same time step as inputs. Console times are in ps.

Q4(a) – 4-bit Full Adder

- Half adder: sum=a^b, carry=a&b. Full adder = 2 HA + OR: sum=a^b^cin, cout=ab+cin(a^b). Yours: (a&b)|(b&cin)|(cin&a), equivalent.

- You wrote: 1-bit dataflow; 4–64-bit behavioral {carry,sum}=a+b+cin; subtractor {bout,diff}=a-b-bin (0−15 → 0001, borrow 1); multiplier 8-bit prod=a*b (9×9=81). 15+15+1=31 → sum 1111, carry 1.

half_adder h1(a,b,s1,c1); half_adder h2(s1,cin,sum,c2); or(carry,c1,c2);

Q4(b) – D FF, JK FF, 4-bit counter

- Latch = level-sensitive; FF = edge-triggered (posedge clk). Sensitivity posedge clk or negedge rst: capture edge + asynchronous reset (drop negedge rst → synchronous).

- <= vs =: non-blocking evaluates all RHS first, updates together → no race between flops; use it in clocked blocks.

- NCLaunch: cds.lib + worklib → ncvlog (compile/syntax) → ncelab (hierarchy, snapshot, test bench top) → ncsim + SimVision (run/waveform).

- Clock always #5 clk=~clk → 10 ns period, rising edges at 5, 15, 25… rst released at 10 ns.

- D FF (d=1@20, 0@30, 1@40, ends 50): edges 15/25/35/45 → q = 0, 1, 0, 1.

- JK (00@10, 10@20, 00@30, 01@40, 11@50, 11@60, ends 70): 15 hold q=0 · 25 set q=1 · 35 hold q=1 · 45 reset q=0 · 55 toggle q=1 · 65 toggle q=0. qbar=~q.

- Counter (rst low 0–10): count=n at edge 5+10n (15→1, 75→7), 1111 at 155, wraps to 0000 at 165, ends 180 ns.

Exp 6 & 7 – Synthesis (RTL Compiler)

- Flow: simulate RTL → SDC → rc -f rc_script.tcl (read_hdl -v2001, elaborate, read_sdc, synthesize -to_mapped -effort medium, report timing/power, write_hdl/write_sdc) → gui_show. -v2001 = ANSI port style.

- SDC: clk 10 ns {0 5}; transition 0.1; uncertainty 0.1; input delay rst 1.0; output delay count 1.0. Library gpdk045 slow.lib.

- Counter slack: 330 ps clk→Q + 1000 ps out delay = 1330 arrival; capture 10000−100 = 9900 → slack 8570 ps. Power 3.021 leak + 8367.397 dyn = 8370.418 nW, 7 cells, SDFFRX4. Simulated counter_4bit; synthesized copy is cntr4bit.

- Pipelined adder: 8 stages × 4-bit = 32-bit; latency 9 edges (~90 ns); trades flops/area/power/latency for a short per-cycle path. GLO-12 = sum_pipe flops that are always 0 (stage k has only 4k live bits); GLO-32 = 24 unused flops deleted. Your 80 ns run shows errors=7 MISMATCHes (expected = x while the 9-deep reference chain fills) and no PASS; a run past ~90 ns is needed for PASS.